EE 271 Digital Design

Final Project Proposal

Name: Martin Gong

Project: Flappy Bird

# PROJECT DESCRIPTION

## From a User Standpoint

This project implements the popular mobile game "Flappy Bird" on the DE1-SoC board using an 8×8 LED matrix display. The player controls a bird (represented by a red LED) that must navigate through a series of moving vertical barriers (represented by green LEDs). The bird is constantly affected by gravity, causing it to fall, and the player must press a button to make the bird "flap" upward.

## User Interaction

- KEY[0]: When pressed, causes the bird to move upward (flap)

- SW[9]: Reset switch to restart the game

- SW[1]: Pause switch to temporarily freeze the game

- SW[0]: When activated, displays the high score on the HEX displays

- HEX Displays (HEX0-HEX5): Show current score during gameplay, or high score when SW[0] is on

- 8×8 LED Matrix: Visual game display showing red LED for the bird's position and green LEDs for barrier columns with gaps to fly through

## Game Mechanics

The bird continuously falls due to gravity simulation. When the player presses KEY[0], the bird moves up one position. Barriers scroll from right to left across the display, and the player must time their button presses to navigate through the gaps. The game ends if the bird collides with a barrier or flies out of bounds (top or bottom of the display). Score increments as barriers successfully pass the bird's position.

# SYSTEM BLOCK DIAGRAM

The system is organized into several major subsystems:

## 1. Input Processing Block

- Clock Divider: Generates multiple clock frequencies from CLOCK_50

- useInput: Debounces KEY[0] input for reliable button detection

- Timing Counters (counter1, counter2, counter3): Generate enable signals for different game speeds

## 2. Bird Control Subsystem

- Eight normalLight modules: Control bird vertical position (one per LED row)

- One centerLight module: Handles middle position with initial spawn logic

- bounds module: Detects out-of-bounds conditions (top/bottom)

- Outputs: red_array[7:0][6] representing bird position in column 6

## 3. Barrier Generation Subsystem

- startColumn: Generates random barrier patterns with gaps for the bird to pass through

- Eight normalColumn modules: Shift barriers left across display (columns 0-7)

- Outputs: green_array[7:0][7:0] representing all barrier positions

## 4. Collision Detection

- collision module: Compares red_array and green_array to detect overlaps

- Function: Signals game over when bird position matches barrier position

## 5. Scoring Subsystem

- score module: Detects when barriers successfully pass bird position

- Three increment modules: Chain together to create 4-digit decimal counter (0-9999)

- Display: Outputs to HEX displays

## 6. Display and Pause Subsystem

- pauseGame: Overlays pause pattern on display when SW[1] is active

- highscore: Stores best score and displays it when SW[0] is on

- matrix_driver: Multiplexes red and green arrays to LED matrix via GPIO pins

# IMPLEMENTATION DETAILS

## Clock Management

All modules operate on a single clock domain (CLOCK_50) with divided clock signals generated by the clock_divider module. The system uses clk[14] as the main game clock for appropriate game speed. Enable signals from counter modules control update rates for different game elements:

- input2: Controls gravity frequency (bird falling rate)

- enable2: Controls barrier scrolling speed

- enable3: Controls new barrier generation rate

## Data Structure

The design uses 2D array structures for both red (bird) and green (barriers) LEDs:

- red_array[7:0][7:0]: 8×8 array where only column [6] is used for bird position

- green_array[7:0][7:0]: Full 8×8 array representing barrier columns

This structure makes collision detection straightforward through bitwise comparison of red_array[row][6] with green_array[row][6].

## Finite State Machine Design

Individual LED positions are controlled by FSM modules (normalLight, centerLight) that respond to:

- Gravity signals: Causing downward movement

- User input: Causing upward movement

- Adjacent LED states: To create continuous bird movement

## Reset and Game Over

The system uses a compound reset signal: (reset | out_of_bounds | dead) that triggers when:

- SW[9] is pressed: Manual reset

- Bird goes out of bounds: Top or bottom

- Bird collides with a barrier: Collision detected

## GPIO Pin Assignment

The LED matrix is driven through GPIO_0[35:0] with the following assignments:

- GPIO_0[27:20]: Red LED drivers (8 bits)

- GPIO_0[35:28]: Green LED drivers (8 bits)

- GPIO_0[19:12]: Row sink control for multiplexing (8 bits)

# KEY FEATURES

- Smooth Gravity Simulation: Adjustable falling rate using counter-based timing

- Responsive Controls: Debounced input ensures reliable button response

- Progressive Difficulty: Continuous barrier generation keeps game challenging

- Score Tracking: Decimal score display up to 9999 with high score memory

- Pause Functionality: Game can be paused without losing state

- Visual Feedback: Clear LED matrix display distinguishes bird (red) from barriers (green)

- Robust Design: All modules follow single-clock methodology for reliable timing

🎮 Top-Level Module

DE1_SoC.sv

The main controller that connects everything together. Think of it as the "game manager" that:

- Sets up the clock (slows 50 MHz down to ~1526 Hz)

- Connects the bird, pipes, collision detection, and scoring

- Sends pixel data to the LED matrix

- Handles reset and game over states

🐦 Bird Modules

birdLight.sv

Controls the bird's vertical position on column 12 of the LED matrix.

- Contains a counter that determines how fast the bird moves

- Instantiates 15 individual light modules (one per row)

- Outputs to GrnPixels (green LEDs)

centerLight.sv

The bird's starting position (row 7, middle of screen).

- Starts in the on state after reset

- Moves up when up is pressed, falls down otherwise

- Only one of these exists (the bird's spawn point)

normalLight.sv

Used for rows 1-6 and 8-13 (all rows except top, bottom, and center).

- Starts off after reset

- Turns on when bird arrives from above (falling) or below (jumping)

- Turns off when bird leaves

topLight.sv

Special case for row 14 (top of screen).

- When bird is here and presses up, it stays here (can't go higher)

- Prevents bird from going off the top of the screen

bottomLight.sv

Special case for row 0 (bottom of screen).

- When bird is here and NOT pressing up, it stays here (can't fall lower)

- Prevents bird from going off the bottom of the screen

🚧 Pipe Modules

newPipe.sv

Generates random pipe patterns offscreen.

- Uses a 3-bit LFSR (linear feedback shift register) for pseudo-randomness

- Creates pipe patterns with gaps at different heights

- Patterns look like 111111100000111 (1s are pipe, 0s are gap)

pipeColumn.sv

One column of the pipe display (15 instances total).

- Acts like a shift register

- Each column takes the previous column's pattern when move triggers

- Column 1 gets new pipes, columns shift left toward column 15

- Freezes position when gameOver is true

💥 Game Logic Modules

collisionCheck.sv

Detects if the bird hits a pipe and awards points.

- Compares bird position (birdloc) with pipe position (column3) at column 12

- If (birdloc & column3) != 0, there's overlap → loser = 1

- If pipe exists and no collision → point = 1 (score!)

- Has its own timing counter to avoid multiple points per pipe

pointDisplay.sv

A single digit of the score counter (0-9).

- Counts up when upCount pulse arrives

- Rolls over from 9 to 0 and sends carry to nextHex

- Outputs to 7-segment display

- Three instances: ones, tens, hundreds

seg7.sv (seg7loser)

Displays "L" on HEX0 when player loses.

- Simple combinational logic

- Shows "L" when lose = 1, blank otherwise

⏱️ Timing & Utility Modules

clock_divider.sv

Divides the 50 MHz FPGA clock into slower frequencies.

- Simple counter that increments every clock cycle

- Each bit is half the frequency of the previous

- divided_clocks[14] ≈ 1526 Hz (used as game clock)

slowDown.sv

Creates periodic timing pulses for game events.

- Counts up to speed value, then outputs a single pulse

- Two instances:

  - createPipe: speed = 15 (slower, for generating new pipes)

  - movePipe: speed = 3 (faster, for scrolling pipes left)

metaFilter.sv

Prevents metastability from button inputs.

- Two flip-flops in series (synchronizer)

- Ensures clean signal from asynchronous button presses

- Used for reset (KEY[3]) and jump (KEY[0])

LEDDriver.sv

Interfaces with the external 16x16 LED matrix.

- Multiplexes rows (only one row lit at a time, but fast enough to look solid)

- Takes RedPixels and GrnPixels arrays

- Outputs row select and column data on GPIO_1

DE1_SoC (top level)

├── clock_divider          - Generates slow game clock

├── metaFilter (x2)        - Button input filtering

│   ├── rst                - Reset button

│   └── keyPress           - Jump button

├── LEDDriver              - LED matrix output

├── birdLight              - Bird controller

│   ├── topLight           - Row 14 (ceiling)

│   ├── normalLight (x12)  - Rows 1-6, 8-13

│   ├── centerLight        - Row 7 (spawn point)

│   └── bottomLight        - Row 0 (floor)

├── slowDown (x2)          - Game timing

│   ├── createPipe         - New pipe timing

│   └── movePipe           - Pipe scroll timing

├── newPipe                - Random pipe generator

│   └── lfsr3              - Random number generator

├── pipeColumn (x15)       - Pipe shift register

├── collisionCheck         - Hit detection & scoring

├── seg7loser              - "L" for lose display

└── pointDisplay (x3)      - Score digits

├── ones               - HEX3

├── tens               - HEX4

└── hundz              - HEX5

Game Flow Summary

- Reset → Bird spawns at center (row 7, column 12)

- Every frame → Bird falls one row (gravity)

- Jump pressed → Bird moves up one row

- Pipe timer → New random pipe enters from right

- Move timer → All pipes shift one column left

- Bird reaches column 12 pipe → Check collision

  - Gap → Score +1, keep playing

  - Hit pipe → Game over, show "L"

- Game over → Everything freezes until reset