EE 271 Lab 8: Final Project Proposal Flappy Bird on FPGA [Your Name] November 21, 2025 1 Project Overview This project proposes the implementation of a ”Flappy Bird” clone on the Altera DE1 Development Board. The system will utilize the Cyclone V FPGA to handle all game logic, physics simulations, and video signal generation. The output will be displayed on a standard VGA monitor at a resolution of 640x480. The objective is to create a responsive, real-time skill game where the player controls a bird navigating through moving obstacles (pipes). The project demonstrates mastery of digital logic design, including finite state machines (FSMs), linear feedback shift registers (LFSR) for pseudo-randomness, VGA timing generation, and module synchronization. 2 User Interaction The user will interact with the hardware via the board’s push-buttons and switches, receiving visual feedback through the VGA monitor and the 7-segment displays. 2.1 Inputs (Controls) • KEY[0] (Fly): The primary game control. Pressing this button causes the bird to ”flap” and gain altitude against simulated gravity. • SW[9] (Game Reset): A high signal on this switch resets the game state (bird position, obstacle positions, score) to the initial state, allowing for a new game to begin after a ”Game Over”. • SW[8] (System Reset): Resets the clock divider and the VGA video driver. This is a hard reset for the display synchronization. 2.2 Outputs (Feedback) • VGA Monitor: Displays the game graphics, including: – The Bird (controlled by the player). 1 – Moving Obstacles (pipes with gaps). – Background color (blue for sky, etc.). • HEX0 – HEX3 (Scoreboard): The current score is displayed in decimal format on the four 7-segment displays. The score increments as the bird successfully passes obstacles. 3 System Architecture The system is synchronized to the board’s 50 MHz clock (‘CLOCK_50‘). A clock divider generates slower enabling signals for game physics to ensure playable speeds. 3.1 Core Modules • VideoDriver: Provided module. Generates VGA timing signals (HS, VS) and current pixel coordinates (x, y). • Game Logic Modules: – Bird: Maintains the vertical position of the bird, applying gravity and lift based on ‘KEY[0]‘. – Obstacle (x3): Manages the horizontal position of pipes. Uses an internal LFSR to generate pseudo-random gap heights when obstacles wrap around the screen. – CollisionDetector: Compares the bird’s bounding box with the obstacles’ bounding boxes. Asserts a collision signal if an overlap is detected. • ScoreCounter: Tracks the player’s score based on obstacles passed and the game state. • VideoGame (Pixel Mux): The visual combiner. It takes the state of all game objects and the current VGA x, y coordinates to determine the RGB color for the current pixel.